Handy Electronic Formulas

Transformers & Coupling

Important- voltage is multiplied or divided directly by the transformer ratio, but impedance is multiplied or divided by the ratio squared. Remember that transformers are frequency and level sensitive, and that measurement conditions should match operating conditions for accurate results.

For mutual inductance, measure the inductance of the primary and secondary in series, and then interchange the connections of one winding for a second reading. Apply the equation below:

M=14Lseries+−Lseries−

For coupling, measure the primary and secondary separately then apply the equation below:

k=MLpLs

k is the coefficient of coupling, zero to one.

The primary leakage inductance is given by:

Lpleak=(1−k)∗Lp

If the secondary is shorted, the resulting primary inductance (Lpss) is said to be given by:

Lpss=(1−k2)∗Lp

The formula does not take into account the resistance of the secondary. It is approximate and should not be rearranged to determine k. Use the first two formulas instead. Note that Lpss is not the primary leakage inductance as sometimes claimed. The non-leakage portion of the primary or secondary inductance is obtained by multiplying the measured inductance by k. The above two formulas can also be applied to the secondary. In general, that which is not mutual inductance must be leakage inductance, but resistive losses and capacitive effects will prevent perfect agreement between the formulas. Refer to a text containing the general coupling solution for more exact results.

Inductors

To use the AL value for toroids, pot cores, or other commercial cores, the value must be expressed in units matching the equations below, i.e., if the AL value is expressed in H/100 turns, use the center equation. L is the inductance, N is the number of turns.

N=LnHAL	N=100∗LuHAL	N=1000∗LmHAL

Q=ωLseriesRseries

Lparallel=Lseries(1+1Q2)

Rparallel=Rseries(1+Q2)

L=Vamps/sec where L is the inductance in henries and V is the applied voltage.

For single layer solenoid wound inductors where a is the coil radius in inches, b is the length in inches, and N is the number of turns, apply the formula below:

LuH=a2N29a+10b

Impedance & Loss

Z=Rseries2+Xcseries−Xlseries2

where,

Xlseries=2πFLseries

and,

Xcseries=12πFCseries

Phase angle

θ=tan−1XseriesRseries

Loss factor

D=tanδ=absRseriesXseries=ωRseriesCseries=1ωCparallelRparallel also, D=1Q

Series to parallel

Cparallel=Cseries11+D2

Parallel to series

Cseries=Cparallel1+D2

Note that if D is less than 0.01, the difference between series and parallel values will be negligible.

If N identical capacitors are paralleled, the capacitance will be 10N and D will be unchanged. Here are the full formulas for paralleled capacitors with significant dissipation factors:

DT=C1D1(1+D22)+C2D2(1+D12)C1(1+D22)+C2(1+D12)

CTS=C2+C1−C2C1(D1−D2)2C1(1+D22)+C2(1+D12)

For more than two capacitors, convert from series to parallel form, and then to ESR at any frequency. Combine the components in the usual fashion, convert from parallel to series, and compute the dissipation factor.

Use the following formula for capacitance between the plates where E=8.85418x10-12, A is the area in meters2, V is the voltage, and S is the distance between the plates in meters. Multiply E by the dielectric constant of the dielectric material.

C=AESFarads

Miscellaneous relationships where C is in farads, Q is coulombs, V is volts, and W is energy. Note that these formulas apply to static conditions, or to vanishingly small time periods only:

C=QV farads	I=Qt amps	V=WQ volts	I=CΔvΔt amps	W=12CV2 joules

Force between the two capacitor plates:

F=AEV22S2Newtons

RC

τ=RC

VCt=E1−e−tτ charging

VCt=Ee−tτ discharging

Rule of thumb for oscilloscope performance:

Trisetime(10%−90%)∗BW−3dB=0.35

Series Resonance

For reasonably high values of Q, the same formulas can be used for both series and parallel tanks. Consult other references for low Q parallel tanks. If the coil resistance is held constant, sharpness of resonance will increase by making the ratio of L to C large. If the Q of the coil remains constant, the ratio of L to C will have no effect on the sharpness of resonance. IOW, with good (low D) capacitors the Q of the coil is usually the controlling factor.

FR=12πLC

To convert C to L or L to C with the exact same impedance at a given frequency (resonance):

L=1ω2C	C=1ω2L

For inductively coupled tanks, maximum selectivity occurs with the least possible coupling. Maximum voltage output occurs when:

ωM=RpRs or for capacitively coupled tanks, 1ωC=RpRs

For Rp >> Lp, the response at resonance is:

VoltageacrossSecondaryVoltageappliedto_Primary=ωMRpRs+(ωM)2∗ωLs

and,

EffectiveActual=11+(ωM)2RsRp

Amplifiers

A linear push-pull power amplifier can be 78% efficient at best, 59% or lower at worst. The power dissipation is maximum at intermediate levels. Power dissipation is given below, where Iout is peak, and Vcc is one supply rail or Vtotal/2. Symmetrical supplies are assumed.

Pd=(2π∗VCC∗IOUT)−(RL2∗IOUT2)

Note that the formula is nothing more than power in minus power out.

# RMS Currents for Various Waveforms

All waveforms except the simple sine wave are unipolar above (or below) ground. The formulas yield RMS current for peak current value inputs, where f is the frequency in Hz and T is the duration of the pulse in seconds.

Sine wave:	Ipk2

Sinewave, rectified, full wave:	Ipk2

Sinewave, rectified, half wave:	Ipk2

Sinewave, offset by DC current:	IDC1+Ipk22	(Idc > Ipk)

Half-sinusoids (unipolar), T duration:	IpkfT2

Squarewaves, T duration:	IpkfT

Sawtooth (unipolar), T duration:	IpkfT3	(use for SMPS design)

Trapezoid (unipolar), T duration, =rise=fall:	Ipkf(2δ+3T)3

AC Coupled True RMS Meters

For true RMS results that include a DC component, measure the AC true RMS value, the DC value average over many cycles and combine using:

TRMS=ac2+dc2

Noise

When uncorrelated noise sources are added together, the total power is equal to the sum of the individual powers. Therefore uncorrelated noise voltages can be added by taking the square root of the sum of the squares of the individual noise voltages.

Vtotal=V12+V22.

Two correlated noise sources can be added by using:

Vtotal=V12+V22+2γV1V2

where  is a correlation coefficient that can have any value from +1 to -1. When  equals 0, the voltages are uncorrelated; when  equals 1, the voltages are totally correlated.

RMS-to-Average Ratios for Gaussian noise:

RMSaverage=π2=1.25or 1.96 dB

Expected Number of Zero-axis Crossings

For "white" random noise with upper and lower frequency limits of f2 and f1, respectively, the expected number of zero crossings per second of either positive-going or negative-going sense is:

N=f23−f133(f2−f1)=13(f22+f1f2+f12)

Expected Number of Maxima

For "white" random noise with upper and lower frequency limits as above, the expected number of maxima per second is:

M=3(f25−f15)5(f23−f13)

If f1 = 0 (low pass filter), M = 0.775 f2.

For narrow band noise (f1f2), M(f1+f2)/2.

The number of maxima per second of the absolute value is double the values of M given above.

Circuit Law Review

The algebraic sum of currents into a node at any instant is zero. (Kirchoff’s Current Law, KCL)

From the above, The sum of currents entering a node equals the sum of currents leaving the node.

The algebraic sum of voltages around a closed loop at any instant is zero. (Kirchoff’s Voltage Law, KCV)

From the above, the sum of voltage rises around a closed loop at any instant is equal to the sum of voltage drops around the loop at the same instant.

Miscellaneous Relationships, Constants and Conversions

ω=2πF radians per second

Slew rate at any given point on a sine wave, with the maximum at zero phase angle (cosine=1):

Slew=VPEAK∗ω∗COS(θ) Volts/Second

For exponential growth or decay, positive k gives growth, negative k gives decay. t is time. For the case where the value doubles every 10 units, say FET op-amp input current vs. temperature, k=0.0693147:

Q(t)=Q0ekt

e=2.71828182846

VOUT=R2R1+R2∗VIN for unloaded voltage dividers (R1 is the upper resistor)

VOUT=R2R1+R2+R1∗R2RLOAD∗VIN for voltage dividers with a resistive load

1 joule = 1 wattsecond = 1 Newton-meter = .0002778 watthour

1 amp-hour = 3600 coulombs

1 pascal = 1 N/m2 = 1.45038E-4 PSI

4.44822 Newtons = 1 lb.

9.80665 Newtons = 1 kg force

0.224809 lbs. = 1 Newton

1 Mpa = 10.197621 kgf/cm^2

E0=8.85418781x10-12 F/m

References:

Instrument Notes, General Radio Corp., IN-103, June 1963

Noise Reduction Techniques in Electronic Systems, Ott, ISBN 0-471-85068-3

Radio Engineers' Handbook, Terman, 1943

Ness Engineering technical data

Various standard engineering & math textbooks